Pico 2 W Script Menu

Complete Setup & User Guide

Raspberry Pi Pico 2 WH  ·  Waveshare LCD-0.96  ·  MicroPython

# 1. What You Need

## 1.1  Hardware

| Pico 2 WH | Raspberry Pi Pico 2 W with pre-soldered headers (the "H" variant). The WH saves soldering the GPIO header yourself. |
| --- | --- |
| LCD Screen | Waveshare Pico-LCD-0.96 — 160×80 pixel ST7735S display. Plugs directly onto the Pico GPIO header with no wiring. |
| USB Cable | Micro-USB cable that supports data (not just charging). Many cheap cables are charge-only — use one you know works for data. |
| Computer | Windows, Mac, or Linux PC to flash the firmware and copy files. |

## 1.2  Software (all free)

| MicroPython | The firmware that runs on the Pico. Download the UF2 file from micropython.org. |
| --- | --- |
| Python 3 | Required to run mpremote. Install from python.org if not already installed. |
| mpremote | Command-line tool to copy files to the Pico. Installed via pip. |
| main.py | The script menu firmware file provided with this guide. |
| st7735 library | LCD driver. Installed directly onto the Pico via mpremote mip. |

The Waveshare Pico-LCD-0.96 plugs directly on top of the Pico 2 WH — all pins are hardwired. No breadboard, no jumper wires, no soldering required.

# 2. Flash MicroPython Firmware

This only needs to be done once. If MicroPython is already on your Pico, skip to Section 3.

- Download the MicroPython UF2 file for Pico 2 W:

Go to: https://micropython.org/download/RPI_PICO2_W/ — download the latest .uf2 file

- Put the Pico into bootloader mode:

- Hold down the BOOTSEL button on the Pico (small white button near the USB port)

- While holding BOOTSEL, plug the USB cable into your computer

- Release BOOTSEL after plugging in

- A drive called RPI-RP2 will appear on your computer, like a USB flash drive

- Flash the firmware:

- Drag and drop the .uf2 file you downloaded onto the RPI-RP2 drive

- The Pico will automatically reboot — the RPI-RP2 drive disappears

- MicroPython is now installed

⚠  If RPI-RP2 does not appear, try a different USB cable. Many cables are charge-only and cannot transfer data.

# 3. Install mpremote

mpremote is the tool used to copy files from your computer onto the Pico. Run these commands in a terminal (Command Prompt on Windows, Terminal on Mac/Linux).

Important: run these commands in your system terminal, NOT inside a Python >>> prompt.

- Check Python is installed by running:

python --version

You should see something like Python 3.11.x. If not, install Python from python.org first.

- Install mpremote:

pip install mpremote

- Verify it installed correctly:

mpremote --version

⚠  On some systems you may need to use "python3" and "pip3" instead of "python" and "pip".

# 4. Install the LCD Driver

The LCD requires a driver library. This is installed directly onto the Pico over USB using mpremote. Make sure the Pico is plugged in before running this.

- Plug in your Pico via USB.

- Run this command in your terminal:

mpremote mip install st7735

mpremote will connect to the Pico and download the driver onto it. You should see output confirming the install completed.

⚠  If mpremote cannot find the Pico, try unplugging and replugging the USB cable. On Windows, check Device Manager to confirm the Pico appears as a COM port.

# 5. Copy main.py to the Pico

main.py is the firmware that runs the menu, LCD display, WiFi access point, web dashboard, games, and HID macros. This file should be saved on your computer first.

- Save main.py to a folder on your computer (e.g. Desktop or Downloads).

- Open a terminal and navigate to the folder where you saved main.py:

cd Desktop

(Replace "Desktop" with wherever you saved the file.)

- Copy main.py to the Pico:

mpremote cp main.py :main.py

- The Pico will automatically run main.py on every boot. To trigger it now without rebooting:

mpremote run main.py

The colon (:) in ":main.py" means "on the Pico". Without it, the file stays on your computer.

⚠  If you update main.py later (e.g. adding new macros), just run "mpremote cp main.py :main.py" again to overwrite it.

# 6. First Boot

With main.py on the Pico and the Waveshare LCD plugged on top, power it on. Here is what you will see:

### Boot Animation

- Stars radiate outward from the centre of the screen

- The text "PICO MENU" builds up letter by letter in cycling colours

- A cyan bar sweeps down the screen, then cuts to black

- The main menu appears

### Main Menu

The menu shows all available scripts and games. Use the joystick to scroll and KEY_A to select.

The boot animation takes about 3–4 seconds. It plays every time the Pico powers on.

# 7. Controls

The Waveshare LCD board has a joystick (4 directions) and two buttons — KEY_A and KEY_B.

| Input | Context | Action |
| --- | --- | --- |
| JOY UP | Main menu | Scroll up through menu items |
| JOY DOWN | Main menu | Scroll down through menu items |
| JOY UP | Settings menu | Scroll up through settings |
| JOY DOWN | Settings menu | Scroll down through settings |
| JOY LEFT | Brightness | Decrease brightness by 10% |
| JOY RIGHT | Brightness | Increase brightness by 10% |
| JOY UP | Pong | Move your paddle up |
| JOY DOWN | Pong | Move your paddle down |
| JOY UP/DN/L/R | Snake | Change snake direction |
| KEY_A | Any menu | Select highlighted item |
| KEY_A/B | Brightness | Confirm and save brightness |
| KEY_B | Settings menu | Go back to main menu |
| KEY_B | Running script | Stop / quit |
| KEY_B | Pong / Snake | Quit game back to menu |

# 8. Menu Items

## 8.1  Settings

Opens a sub-menu with device settings. KEY_B exits back to the main menu.

### WiFi Dashboard

Shows the WiFi access point credentials and IP address on screen. Connect to the WiFi network shown to access the web dashboard.

### Brightness

Adjust the LCD backlight brightness in 10% steps using JOY LEFT (dimmer) and JOY RIGHT (brighter). The value is saved to the Pico automatically — it will be remembered after a reboot.

Brightness is saved to a file called brightness.txt on the Pico flash storage. On first boot before any adjustment it defaults to 100%.

## 8.2  HID Macros

These macros use the USB HID keyboard interface to type keystrokes directly into whatever computer the Pico is plugged into, just as if someone was typing on a keyboard.

### Macro: GitHub

Opens the Windows Run dialog (WIN+R), types https://github.com, and presses Enter. Opens GitHub in the default browser.

### Macro: YouTube

Opens the Windows Run dialog, types https://youtube.com, and presses Enter. Opens YouTube in the default browser.

### Macro: Cmd

Opens a Command Prompt and types "Hello from Pico!". Useful as a test that HID is working.

### Macro: Lock PC

Sends the WIN+L keyboard shortcut to lock the Windows PC immediately.

HID macros only work when the Pico is connected to a computer via USB. They will not work if the Pico is powered from a USB charger.

⚠  MicroPython's built-in USB HID module is used — no extra libraries or CircuitPython needed.

## 8.3  Games

### Pong

- You are the right paddle. CPU controls the left paddle.

- Use JOY UP / JOY DOWN to move your paddle.

- First to 7 points wins.

- The ball speeds up slightly every time it hits a paddle.

- KEY_B quits back to the menu at any time.

### Snake

- Control the snake using all four joystick directions.

- Eat the red food squares to grow and score 10 points each.

- The snake speeds up every 5 food eaten.

- Hitting a wall or your own body ends the game.

- Your score is displayed on the game over screen.

- KEY_B quits back to the menu at any time.

# 9. WiFi Dashboard

The Pico creates its own WiFi access point. Any device (phone, tablet, laptop) can connect to it and open the web dashboard.

## 9.1  Connecting

- On your phone or laptop, open WiFi settings.

- Connect to the network named PicoMenu.

- Enter the password: pico1234

- Open a browser and go to: http://192.168.4.1

The dashboard works on any browser. No internet connection is required — the Pico serves the page itself.

## 9.2  Dashboard Features

### Status Card

Shows whether a script is currently running, the script name, a progress bar, and a live detail line.

### System Stats Card

Updates every 3 seconds and shows:

- Uptime — time since the Pico last booted

- Free RAM — memory available in kilobytes, with a usage bar

- WiFi RSSI — signal strength of the access point in dBm

### Scripts and Macros Card

Buttons for each macro and WiFi Dashboard. Click a button to trigger it remotely — the Pico will execute it just as if you pressed KEY_A on the device.

Games (Pong and Snake) do not appear in the dashboard — they require the physical joystick.

### Custom Macro Builder

Type any text into the text box and click "Send to Pico". The Pico will type it out via USB HID keyboard — useful for sending custom commands, URLs, or text without editing any code.

- Limited to 200 characters per send.

- If the Pico is busy running something, it will return a "busy" error.

### Stop Button

Stops whatever script is currently running on the Pico, equivalent to pressing KEY_B on the device.

### Live Log

A scrolling log of all events — script starts, completions, errors, and web triggers. Useful for debugging.

# 10. Adding Your Own Macros

Open main.py in any text editor and find the PAYLOADS list near the middle of the file. Add a new entry like this:

{

"name": "My Macro",

"steps": [

("combo",  [KC_WIN, KC_R]),   # press WIN+R

("delay",  500),              # wait 500ms

("string", "notepad"),        # type text

("key",    KC_ENTER),         # press Enter

],

},

Then add a wrapper function and menu entry below the PAYLOADS list:

def macro_mine():  run_macro(PAYLOADS[4])   # use the correct index

{"name": "My Macro", "fn": macro_mine},     # add to MENU list

### Available Step Types

| ("key", KC_X) | Press and release a single key. E.g. ("key", KC_ENTER) |
| --- | --- |
| ("combo", [K1, K2]) | Hold modifier key(s) and tap the last key. E.g. ("combo", [KC_WIN, KC_R]) |
| ("string", "text") | Type a string of characters one by one. Works with a-z, A-Z, 0-9, and common punctuation. |
| ("delay", ms) | Wait for the given number of milliseconds before the next step. |

### Common Key Codes

| KC_WIN | Windows key |
| --- | --- |
| KC_CTRL | Left Control |
| KC_SHIFT | Left Shift |
| KC_ALT | Left Alt |
| KC_ENTER | Enter / Return |
| KC_ESC | Escape |
| KC_TAB | Tab |
| KC_L | L key (used in WIN+L to lock PC) |
| KC_R | R key (used in WIN+R Run dialog) |

# 11. Adjusting Settings in Code

A few settings at the top of main.py can be changed to customise the device:

| AP_SSID | The WiFi network name. Default: PicoMenu |
| --- | --- |
| AP_PASSWORD | The WiFi password. Default: pico1234 |
| AP_IP | The IP address of the dashboard. Default: 192.168.4.1 |
| BRIGHTNESS | The default brightness on first boot (0–100). Default: 100 |
| SERVER_PORT | The web server port. Default: 80 |

⚠  After changing main.py, you must copy it back to the Pico with: mpremote cp main.py :main.py

# 12. Troubleshooting

| Screen stays blank | Check the LCD board is fully seated on the GPIO header. All pins must be connected. Try repowering. |
| --- | --- |
| RPI-RP2 drive not appearing | Use a different USB cable — many cables are charge-only. Hold BOOTSEL before plugging in. |
| mpremote: device not found | Unplug and replug the Pico. On Windows check Device Manager for a COM port. Try running: mpremote devs |
| st7735 install fails | Ensure the Pico is running MicroPython (not CircuitPython) and is connected to internet via the Pico W WiFi before running mip install — or install from a PC with mpremote mip install st7735. |
| HID macros do nothing | The Pico must be connected to a computer via USB, not a charger. Check that MicroPython 1.23+ is installed as older versions lack the usb.device.hid module. |
| WiFi network not appearing | Wait 10 seconds after boot for the AP to start. The WiFi screen in Settings shows the IP when ready. |
| Dashboard wont load | Make sure you are connected to PicoMenu WiFi, not your home network. Use http:// not https://. |
| Brightness not saving | The Pico needs a brief moment to write the file. Always press KEY_A or KEY_B to confirm before leaving the brightness screen. |
| Game is very slow | Ensure no other scripts are running. Restart the Pico. If using mpremote, disconnect it as it can interfere. |

# 13. Files on the Pico

After setup, the following files will be stored on the Pico's flash memory:

| main.py | The main firmware. Runs automatically on every boot. |
| --- | --- |
| brightness.txt | Stores your saved brightness level. Created the first time you adjust brightness. |
| lib/st7735.py | The LCD driver library. Installed by mpremote mip install st7735. |

You can list all files on the Pico by running: mpremote ls

# 14. Quick Reference

### Setup Commands (run once)

pip install mpremote

mpremote mip install st7735

mpremote cp main.py :main.py

### Update Commands (when you change main.py)

mpremote cp main.py :main.py

### Useful mpremote Commands

| mpremote devs | List connected Pico devices |
| --- | --- |
| mpremote ls | List files on the Pico |
| mpremote cp main.py :main.py | Copy main.py to the Pico |
| mpremote run main.py | Run main.py without copying |
| mpremote repl | Open the Python REPL on the Pico |
| mpremote rm :brightness.txt | Delete the saved brightness file (resets to 100%) |

### Default WiFi Credentials

| Network name (SSID) | PicoMenu |
| --- | --- |
| Password | pico1234 |
| Dashboard URL | http://192.168.4.1 |